[image: image1.png]


                 
                      江 苏 极 易 新 材 料  有  限  公  司

          Jiangsu Jiyi New Material Co.,Ltd


MATERIAL SAFETY DATA SHEET
JYANOX-225
1. CHEMICAL PRODUCT AND COMPANY IDENTIFICATION
Product Name: JYANOX –225
Chemical Name: JYANOX-225 is a kind of compound components , including 
JYANOX-1010 and JYANOX-168.
Use: Antioxidant
Producer/Supplier:  Jiangsu Jiyi New Material Co., Ltd.
North of North Ring Road and West  of Dongcheng Road,
Economic Development Zone of Fengxian,Xuzhou City，Jiangsu Province,China 
Emergency Phone: +86 10 8938 9018
2.COMPOSITION, INFORMATION ON INGREDIENTS
  Product Name: JYANOX –225
  Content: JYANOX-1010 and JYANOX-168
C.A.S.NO.: JYANOX-1010 [6683-19-8], JYANOX-168 [31570-04-4] 
3. HAZARDS IDENTIFICATION
  Classification of Danger :General Solid Chemical 
Path of Intrusion : Inhale, swallow or skin contact 
Danger to Health : This chemical is low in poisonousness. white mice did not reveal danger to cause cancer, neither any effect on offspring (antioxidant  JYANOX-1010 , LD50>/= 
5000mg/Kg ; antioxidant JYANOX-168 , LD50>/= 6000mg/Kg). The suggested dosage 
of this chemical is 0.1%~0.8% (varies according to different resins to be processed). 
Maximum dosage in food packaging </= 0.5%.  
Danger to Environment :  Long-term effect on environment has not been evaluated.
Danger of Explosion: Therefore should be insulated from fire. Any manufacturing equipment that contacts this product should have ground connection to prevent static electricity. 
4. FIRST AID MEASURES 
Skin Contact :Use large amount of water and soap to wash the contaminated section. The clothes should be washed before it is weared again .Send to hospital for timely further treatment on cases where there is rash. 
Eye Contact :Use large amount of water to wash for 15~20 minutes. give medical treatment if there is irritation. 
Inhaled :Make sure the patient's nose is open. Move the patient to where there is fresh air. If the patient has difficulty breathing, feed oxygen. 
Swallowed: Feed 2-3 cups of water, and insert fingers to the throat to make the patient to vomit. This should be done with doctor's instruction. Do not feed anything else to patients who are in coma..
5. FIRE-FIGHTING MEASURES
Dangerous Quality :The product should not be heated. Avoid contact with live fire or sparkles.  
Harmful Substances Resulted From Burning : Carbon monoxide, Carbon dioxide , Aromatic compounds , micromolecular carbohydrate compounds and oxidic phosphorus . 
Fire Extinguishing Methods and Chemicals : Enter the scene from the wind-ward side. Do not inhale fume. Fire extinguishing chemicals usable are: chemical powder , sand , foam, 
and fine spray. 
Precautions: When there is excessive fume,  wear masks and other fire extinguishing 
equipments available.  
6. ACCIDENTAL RELEASE MEASURES
Emergency Treatment :Sweep the chemical or vacuum it. Put it in a special container for 
recycling. 
Treatment: This chemical is not poisonous waste, and will not pose any danger to human if 
spilled. Long time effect on environment has not been evaluated. Suggested treatment is to 
incinerate in a stove for chemical waste. It can also be treated in other ways permitted by the local regulations. 
7. HANDLING and STORAGE
Operation Precautions :Before this chemical is packed or used, the workers should wear 
functional dust masks, and special protective uniforms (long pants and long sleeve shirts), chemical anti-erosion gloves. Keep the work place well ventilated, free of dust, live fire, or sparkles. Change clothes after work. 
Storage Precautions : This chemical should be stored in ventilated, and cool chemical 
warehouses. No Fire, No rain. Keep the packing intact so that it will not be contaminated by dust or decomposed by humidity. The remainder of the product should be packed well .  
8. EXPOSURE CONTROLS, PERSONAL PROTECTION
Maximum Allowed Concentration : The law is not exit in China .  
Solutions : Improve ventilation, such as fans. 
Respiratory System Protection : Wear anti-dust masks or respirators, when working in places where there is chemical powder. 
Eye Protection :Wear anti-chemical goggles or masks. 
Body Protection :Wear special protective uniform (long pants, long sleeve shirts). 
Hand Protection :Wear anti-erosion gloves. 
Other Protection : Eat or drink after shower and change. 
9. PHYSICAL AND CHEMICAL PROPERTIES
Appearance : White crystal powder or granule 
Volatility : </= 0.50% 
Boiling Point : N/A 
Transmittance   425nm  >/= 96%    ；           500nm  >/= 98%  
Content of antioxidant JYANOX-168   45.0~55.0%    
Solubility :Soluble in benzene , hexane , ethyl acetate , insoluble in water .  
Smell and Taste: No smell, no taste. 
10. STABILITY AND REACTIVITY
Stability :This chemical is chemically stable, and will not explode.                   
Incompatibility :Strong acid, strong oxidant. 
Prohibited Environment :Heating, humidity, sparkle or live fire. 
Polymerization : Not possible 
Decomposition Products : Carbon monoxide, Carbon dioxide, Aromatic compounds,   
icromolecular carbohydrate compounds and oxidic phosphorus .   
11. TOXICOLOGICAL INFORMATION
Acute Toxicity : antioxidant JYANOX-1010 , LD50>/= 5000mg/Kg ;  antioxidant JYANOX-168 , LD50>/=6000mg/Kg dosage per Kg weight to cause white half dead. This
chemical is low in poisonousness.  
Irritation :May cause slight irritation to eyes. Avoid eyes or skin  contacting the product .   
Affection of  contact  for a long time : white mice did not reveal danger to cause cancer,
neither any effect on off spring  
12. ECOLOGICAL INFORMATION
Biological Toxicity : Low toxicity to organisms in water. 
13. DISPOSAL CONSIDERATIONS
Nature of Disposal: This chemical is not hazardous 
Method of Disposal: Destroy by fire in an incinerator. And decompose the packaging.  
Recycling of the packaging is prohibited. Disposal treatment should abide by the local laws. 
14. TRANSPORT INFORMATION
Signify :No Fire, No Over Exposure to Sunlight, No rain. 
Means of Transportation : May be packed in paper-plastic bags, lined with plastic bags.  
25Kg/bag. 
Transportation Precautions: Protect the chemical from humidity and fire. 
15. REGULATORY INFORMATION
  <<The Safety Work Law of The People Republic of China >> 
  The Law to The Dangerous Chemicals. (The 344th Law of State Department ) 
16. ADDITIONAL INFORMATION
  The information above is believed to be accurate and represents the best information currently 
  available to us. We assume no liability resulting from its use. Users should make their own 
  investigations to determine the suitability of the information for their particular purposes.
Revision Date: Aug.1st, 2017
Jiangsu Jiyi New Material Co., Ltd.
Reference,
1.) Jiyi’s “Production Safety Operation Regulation”,
2.) Jiyi’s Fire Protection Emergency Management Counterplan,
3.) Jiyi’s Environment Protection Emergency Management Counterplan.

  Address: North of North Ring Road and West of Dongcheng Road, Economic Development Zone of Fengxian, Xuzhou City
  http://www.jiyichem.com/                  Tel: +86-516-89207566                  Fax: +86-516-89207166

_2147483647

